DSP Project

Audio Equalizer

| Name | ID |
| --- | --- |
| Sherif Rafik | 4635 |
| Mahmoud Salem | 4643 |

### Introduction:

### Problem Statement:

It is required to develop audio equalizer using MATLAB. The equalizer function is to vary the gain of each specific band as the user prefers. [ i.e. if the user likes base, he will increase the gain of low frequencies].

### Inputs to the program:

- Wave file name.

- Gain of each of the frequency bands in db.

- Type of filter used (FIR – IIR)

- Output Sample Rate

### Outputs of the program:

- Composite signal is played and can be saved

- Filter analysis results

- Figures of signals in time and frequency domains

### Extras:

- There are 5 built in presets which the user can use on his wav file:

- Pop

- Rock

- Techno

- Party

- Classical

- User can pause and stop the sound file.

### Instructions:

- Load up an audio file

- Input the output sample rate

- Choose the type of the filter (FIR / IIR)

- Adjust the sliders according to the gain of each filter

- Play the output sound

- Save the output sound

- In filter analysis tab, check the analysis results of the filters

### Global variables used:

### Functions used:

- uigetfile: Open file selection dialog box, used for inputting the wav file to the equalizer

- resample: Resample uniform or nonuniform data to new fixed rate, used for resampling the input wav file to the new sampling rate inputted by the user

- audioread: Reads data from an audio file and returns sampled data and sample rate

- butter: Butterworth filter design, used for designing the IIR filters, the function returns the numerator and denominator of the transfer function of the filter

- fir1: Window based FIR filter design, used for designing the FIR filters, the function returns the numerator of the transfer function of the filter

- filter: It filters the input data using a rational transfer function defined by the numerator and the denominator, used in filtering the input wav file

- impz: Impulse response of a digital filter, the function chooses the number of samples and plots the impulse response, used to calculate the impulse response of the filters whether FIR or IIR

- stepz: Step response of a digital filter, the function chooses the number of samples and plots the step response, used to calculate the step response of the filters whether FIR or IIR

- freqz: Frequency response of a digital filter, used to calculate the phase and gain of each filter whether its FIR or IIR

- tf: It returns a transfer function model

- pzmap: Pole-zero plot of a dynamic system, used to plot the poles and zeros of each filter whether FIR or IIR

- fft: Computes the discrete Fourier transform (DFR) using the fast Fourier transform algorithm

- fftshift:  It rearranges the flourier transform by shifting the zero-frequency component to the center of the array

- audioplayer:  Use an audioplayer object to play audio, the object contains properties that enable additional flexibility during playback. For example, you can pause or resume.

- audiowrite:  Writes a matrix of audio data, with a specific sample rate.

### Filter analysis Results:

### IIR filter:

- Poles and Zeros of each filter:

- Impulse response of each filter:

- Step response of each filter:

- Phase and gain:

- Filter 0 – 170Hz

- Filter 170 – 310Hz

- Filter 310 – 600Hz

- Filter 600 – 1000Hz

- Filter 1 – 3KHz

- Filter 3 – 6KHz

- Filter 6 – 12KHz

- Filter 12 – 14KHz

- Filter 14 – 16KHz

### FIR filter:

- Poles and Zeros of each filter:

- Impulse response of each filter:

- Step response of each filter:

- Phase and gain:

- Filter 0 – 170Hz

- Filter 170 – 310Hz

- Filter 310 – 600Hz

- Filter 600 – 1000Hz

- Filter 1 – 3KHz

- Filter 3 – 6KHz

- Filter 6 – 12KHz

- Filter 12 – 14KHz

- Filter 14 – 16KHz

### Sample runs:

- Sample run 1:

- Output sample rate = 44100

- IIR Filter

- Original Signal

- Composite Signal

- Sample run 2:

- Output sample rate = 44100

- FIR Filter

- Original signal:

- Composite Signal

- Output signal in case if doubling output sample rate or decreasing it to half

- Sample run 3:

- Output sample rate = 33000

- IIR Filter

- Original signal

- Composite signal

- Sample run 4:

- Output sample rate = 66000

- IIR Filter

- Original signal

- Composite signal

- Sample run 5:

- Output sample rate = 132000

- IIR Filter

- Original signal

- Composite signal